## Fleet Standards and Training

## Aircraft Operation Manual

### Dornier Do-328 Turbo Prop

##### Technical Specifications

#### Exterior Dimensions

Wingspan				68 feet 10 inches

Overall Length			69 feet 10 inches

Tail Height				23 feet 9 inches

Interior Dimensions

32 Pax		31” Pitch

Specifications

Engines – Pratt & Whitney PW 119B (2)

Props – 6 Blade Composite Hartzell, 11 ft 10 in

Max HP – 2,180 lbs each

Max Fuel Capacity – 7,557 lbs

Max Takeoff Weight – 30,843 lb

Max Range – 1,000 nautical miles

Service Ceiling – 25,000 feet

Max Cruise Speed – 335 kts

Cruise Speed – 270 to 310 kts

Cruise Altitude – FL180 to FL230

Runway Length Takeoff – 3,570 feet

Runway Length Landing – 3,820 feet

Clean Stall – 100 kts

Dirty Stall (flaps & gear) – 84 kts

V1 speed –	112

Vr speed –	114

V2 speed –	123

Vref –	132

Vmo -	350 kts

(Please note that V speeds vary with the weight of you’re A/C.  These V speeds were based on 30,843 lbs takeoff weight)

###### Aircraft Operation

PRE-TAXI:

Cabin Check List

Push Back

Start Engines

PRE-TAKE-OFF;

Taxi to Assigned Runway

Flaps 5 degrees (F7 till 5)

Auto Pilot on (Do not engage functions until airborne).

Set Airspeed-250 Knots

Set Runway Heading

Set Altitude to level cleared

Taxi into position when cleared

TAKE-OFF;

Take-Off Thrust 94% (Not Full Throttle)

V1 112 Knots

Vr 114 Knots

V2 123 Knots

Initial Climb Rate=15 degrees

Positive Rate of Climb-Gear Up

POST TAKE-OFF:

Engage Auto-Throttle

Engage Heading Hold

Engage Altitude Hold

CLIMB:

Climb Rate 1500 to 1800 Ft/Min.

Flaps Up (F5) at 205 kias

Accelerate to 250 kias

Continue Climb, Maintaining  250 kts (N1 will increase from 90% to 96%)

Service Ceiling = FL250

CRUISE:

Cruise Airspeed= 270 to 310 kias (N1 77%-86%)

DESCENT:

Reduce to Idle Speed.

Descent rate=1800 Ft/min

At 10,000 ft Reduce Airspeed to 250 kias

At 12 NM from Touch-Down, reduce Airspeed to 190kias

APPROACH:

Reduce Airspeed to 150 kias-Flaps 5 degrees

If put into a Holding Pattern, Maintain 190 kias -Flaps Up

At Outer Marker Lower Landing Gear

Slow to 120 kias

Flaps to Full (40 degrees) in Increments.

LANDING:

At Nose Wheel Touch-Down, Engage Reverse Pitch

At 60 Knots feather Reverse Pitch to normal

Apply Manual Braking

Exit Runway at 15 Knots

POST-LANDING:

Retract Flaps and Spoilers

Taxi to Terminal when cleared

PARKING:

Parking Brake Set

Engines Shut Down (ctrl+shift+F1)

Perform Final Cabin Check List

Aircraft Designer:  Bill Alderson

Manual Designer:  Chris Smith

Copyright 2000 Cyber Air Virtual Airlines